Unit 1 : Internet Architecture and Network Layer

Part 1

This part covers:

- Internet

- Intranet

- ISP

- ICANN

1. Internet

Definition

The Internet is a worldwide network that connects millions of computers and devices so they can communicate and share information using the TCP/IP protocol.

Simple Definition (Exam):

The Internet is a global network of interconnected computers used for communication and information sharing.

Need of Internet

The Internet is used because it allows people to:

- Share information

- Send emails

- Browse websites

- Watch videos

- Attend online classes

- Shop online

- Make video calls

- Store data in the cloud

How Internet Works

When you open a website:

- Your device sends a request.

- The request goes to your router.

- The router sends it to your ISP.

- The ISP forwards it through the Internet.

- The web server receives the request.

- The server sends the webpage back.

- Your browser displays the webpage.

Diagram

Computer

│

Wi-Fi Router

│

ISP

│

Internet

│

Web Server

│

Website

Features of Internet

- Global communication

- High-speed information sharing

- Resource sharing

- Available 24×7

- Supports multimedia (audio, video, images)

- Easy access to knowledge

Advantages

- Fast communication

- Online education

- Digital banking

- Online shopping

- Entertainment

- Cloud storage

- Social networking

Disadvantages

- Cybercrime

- Virus attacks

- Privacy issues

- Internet addiction

- Fake information

- Data theft

Real-Life Examples

- Google Search

- YouTube

- WhatsApp

- Gmail

- Amazon

- Netflix

Important Exam Points

- Global network

- Uses TCP/IP

- Connects millions of devices

- Information sharing

2. Intranet

An Intranet is a private network that uses Internet technologies but is accessible only to authorized users within an organization.

Simple Definition (Exam):

An Intranet is a private network used inside an organization for secure communication and information sharing.

Need of Intranet

Organizations use an Intranet to:

- Share company documents

- Store employee information

- Communicate internally

- Manage projects

- Share notices and announcements

Working

Employees connect to the company's network.

Only authorized users can log in.

Information remains inside the organization.

Diagram

Employees

│

Company Network

│

Intranet Server

│

Company Database

Features

- Private network

- Secure

- Fast communication

- Controlled access

- Internal file sharing

Advantages

- Better security

- Faster communication

- Easy document sharing

- Low communication cost

- Centralized management

Disadvantages

- Limited access

- Setup cost

- Requires maintenance

- Needs network administrator

Examples

- College ERP Portal

- Company HR Portal

- Hospital Management System

- Bank Employee Portal

Difference between Internet and Intranet

| Internet | Intranet |
| --- | --- |
| Public network | Private network |
| Anyone can use | Only authorized users |
| Worldwide | Organization only |
| Less secure | More secure |
| Uses public servers | Uses private servers |

3. ISP (Internet Service Provider)

Definition

An Internet Service Provider (ISP) is a company that provides Internet access to homes, offices, schools, and businesses.

Simple Definition (Exam):

An ISP is a company that provides Internet services to users.

Functions of ISP

- Provides Internet connection

- Assigns IP addresses

- Provides email services

- Maintains network connectivity

- Offers technical support

- Manages bandwidth

Services Provided by ISP

- Broadband

- Fiber Internet

- Mobile Internet

- Wi-Fi

- Email

- Web hosting

- Domain registration

Working of ISP

User

│

Wi-Fi Router

│

ISP

│

Internet Backbone

│

Web Server

Examples of ISP

India

- JioFiber

- Airtel Xstream Fiber

- BSNL Bharat Fiber

- ACT Fibernet

Advantages

- High-speed Internet

- Technical support

- Reliable connection

- Secure network

- Different Internet plans

Important Exam Points

- Provides Internet

- Assigns IP address

- Connects users to the Internet

- Provides customer support

4. ICANN (Internet Corporation for Assigned Names and Numbers)

Definition

ICANN is an international organization that manages domain names and IP address allocation so that every Internet address remains unique.

Simple Definition (Exam):

ICANN manages domain names and IP addresses on the Internet.

Why ICANN is Needed

Without ICANN:

- Two websites could have the same domain name.

- Internet addresses could conflict.

- Websites might not open correctly.

Functions of ICANN

- Allocates IP addresses

- Manages domain names

- Coordinates DNS root servers

- Approves new domain extensions

- Maintains uniqueness of domain names

Example

When someone registers:

www.mycollege.in

ICANN ensures that no one else can register the exact same domain name.

Popular Domain Extensions

| Extension | Meaning |
| --- | --- |
| .com | Commercial |
| .org | Organization |
| .edu | Educational |
| .gov | Government |
| .net | Network |
| .in | India |

Advantages of ICANN

- Unique domain names

- Organized Internet

- Reliable DNS operation

- Global coordination

- Stable Internet infrastructure

Internet vs Intranet vs ISP vs ICANN

| Feature | Internet | Intranet | ISP | ICANN |
| --- | --- | --- | --- | --- |
| Type | Global network | Private network | Service provider | Internet management organization |
| Purpose | Connects the world | Internal communication | Provides Internet access | Manages domain names and IP addresses |
| Users | Everyone | Organization members | Internet customers | Internet infrastructure |
| Example | Google | College ERP | JioFiber | ICANN |

1 m Questions

- Define Internet.

- Define Intranet.

- What is ISP?

- What is ICANN?

- Give two examples of ISP.

- What is a domain name?

2/4 m Questions

- Explain the Internet with a diagram.

- Explain the Intranet.

- Explain the functions of ISP.

- Explain the functions of ICANN.

- Differentiate between Internet and Intranet.

Quick Revision

- Internet → Global network connecting devices worldwide.

- Intranet → Private network inside an organization.

- ISP → Company that provides Internet access.

- ICANN → Organization that manages domain names and IP addresses.

Part 2

This part covers:

- IPv4

- IPv4 Header Format

- IPv6

- IPv6 Header Format

- Difference between IPv4 and IPv6

1. IPv4 (Internet Protocol Version 4)

Definition

IPv4 (Internet Protocol Version 4) is the fourth version of the Internet Protocol. It is used to identify devices connected to a network using a 32-bit address.

Simple Definition (Exam)

IPv4 is a 32-bit Internet Protocol used to identify devices on a network.

Why IPv4 is Needed

Every device connected to the Internet must have a unique IP address. IPv4 provides this address so that data can reach the correct destination.

Example:

- Mobile phone

- Laptop

- Computer

- Printer

All these devices need an IP address.

Features of IPv4

- 32-bit address

- Divided into 4 octets

- Written in decimal numbers

- Each octet ranges from 0 to 255

- Supports about 4.3 billion IP addresses

IPv4 Format

Example:

192.168.10.25

This address contains 4 parts (octets) separated by dots.

192 .    168 .   10 .     25

↑          ↑           ↑         ↑

Octet  Octet  Octet  Octet

Advantages of IPv4

- Easy to understand

- Widely used

- Supported by almost all devices

- Simple configuration

Disadvantages

- Limited address space

- Address shortage

- Requires NAT (Network Address Translation)

- Less secure than IPv6

2. IPv4 Header Format

Whenever IPv4 sends data, it adds a header before the actual data.

The header contains information needed for routing the packet.

IPv4 Header Diagram

---------------------------------------------------------

| Version | Header Length | Type of Service | Total Length |

---------------------------------------------------------

| Identification | Flags | Fragment Offset |

---------------------------------------------------------

| TTL | Protocol | Header Checksum |

---------------------------------------------------------

| Source IP Address |

---------------------------------------------------------

| Destination IP Address |

---------------------------------------------------------

| Options (Optional) |

---------------------------------------------------------

| Data (Payload) |

---------------------------------------------------------

Important Header Fields

1. Version

- Indicates the IP version.

- For IPv4, the value is 4.

2. Header Length (IHL)

- Specifies the size of the header.

- Minimum size: 20 bytes

3. Type of Service (ToS)

- Specifies packet priority and quality of service.

4. Total Length

- Total size of the packet (header + data).

5. Identification

- Identifies fragmented packets.

6. Flags

- Controls fragmentation of packets.

7. Fragment Offset

- Indicates the position of each fragment.

8. Time To Live (TTL)

- Limits how many routers a packet can pass through.

- Prevents packets from looping forever.

9. Protocol

Specifies which transport layer protocol is used.

Examples:

- TCP

- UDP

- ICMP

10. Header Checksum

- Detects errors in the IPv4 header.

11. Source Address

- Sender's IP address.

Example:

192.168.1.10

12. Destination Address

- Receiver's IP address.

Example:

192.168.1.20

Important Exam Point

Minimum IPv4 Header Size = 20 Bytes

3. IPv6 (Internet Protocol Version 6)

Definition

IPv6 is the latest version of the Internet Protocol. It uses a 128-bit address, providing a much larger address space than IPv4.

Simple Definition (Exam)

IPv6 is a 128-bit Internet Protocol used to overcome the shortage of IPv4 addresses.

Why IPv6 was Developed

IPv4 can provide only about 4.3 billion addresses, which became insufficient as the number of Internet-connected devices increased.

IPv6 was developed to solve this problem.

Features of IPv6

- 128-bit address

- Very large address space

- Faster routing

- Better security

- Supports auto-configuration

- No NAT required

IPv6 Format

Example:

2001:0db8:85a3:0000:0000:8a2e:0370:7334

IPv6 uses 8 groups of hexadecimal numbers separated by colons (:).

Advantages

- Huge address space

- Built-in security (IPsec support)

- Better performance

- Efficient routing

- Supports modern Internet applications

Disadvantages

- More difficult to remember

- Upgrade cost

- Not supported by some older devices

4. IPv6 Header Format

The IPv6 header is simpler than the IPv4 header, making packet processing faster.

IPv6 Header Diagram

----------------------------------------------------------

| Version | Traffic Class | Flow Label |

----------------------------------------------------------

| Payload Length | Next Header | Hop Limit |

----------------------------------------------------------

| Source Address (128 bits) |

----------------------------------------------------------

| Destination Address (128 bits) |

----------------------------------------------------------

| Data |

----------------------------------------------------------

Header Fields

Version

- Indicates the IP version.

- Value = 6

Traffic Class

- Indicates packet priority and quality of service.

Flow Label

- Identifies packets belonging to the same data flow.

Payload Length

- Specifies the size of the data (excluding the header).

Next Header

- Indicates the next protocol or extension header.

Hop Limit

- Similar to TTL in IPv4.

- Limits the number of routers a packet can pass through.

Source Address

- Sender's IPv6 address.

Destination Address

- Receiver's IPv6 address.

Important Exam Point

IPv6 Header Size = 40 Bytes (Fixed)

Difference between IPv4 and IPv6

| IPv4 | IPv6 |
| --- | --- |
| 32-bit address | 128-bit address |
| 4 octets | 8 hexadecimal groups |
| Decimal notation | Hexadecimal notation |
| About 4.3 billion addresses | Extremely large address space |
| Header size: Variable (minimum 20 bytes) | Header size: Fixed 40 bytes |
| Uses NAT | No NAT required |
| Limited security | Better built-in security |
| Uses TTL | Uses Hop Limit |

Memory Trick

IPv4

32 Bits → 4 Parts → Dots (.)

Example:

192.168.1.10

IPv6

128 Bits → 8 Parts → Colons (:)

Example:

2001:db8:85a3::8a2e:370:7334

One-Mark Questions

- Define IPv4.

- Define IPv6.

- How many bits are used in IPv4?

- How many bits are used in IPv6?

- What is the minimum IPv4 header size?

- What is the IPv6 header size?

- What is TTL?

- What is Hop Limit?

Two/Four-Mark Questions

- Explain IPv4.

- Explain IPv6.

- Explain the IPv4 header format.

- Explain the IPv6 header format.

- Compare IPv4 and IPv6.

Important Viva Questions

- Why was IPv6 introduced?

- Which protocol has the larger address space?

- What is the purpose of the TTL field?

- What is the function of the Hop Limit field?

- Why is the IPv6 header simpler than the IPv4 header?

Quick Revision

- IPv4 → 32-bit address, 4 octets, decimal notation, minimum header size 20 bytes.

- IPv6 → 128-bit address, 8 hexadecimal groups, fixed header size 40 bytes.

- TTL (IPv4) and Hop Limit (IPv6) both prevent packets from circulating indefinitely.

- IPv6 provides a much larger address space and improved efficiency compared to IPv4.

Part 3

This part covers:

- Subnet

- Subnet Addressing

- Address Masking (Subnet Mask)

- Supernetting

- ARP (Address Resolution Protocol)

- RARP (Reverse Address Resolution Protocol)

1. Subnet

Definition

A Subnet (Subnetwork) is a smaller network created by dividing a large network into multiple smaller networks.

Simple Definition (Exam)

A subnet is a smaller logical network formed by dividing a large network.

Why is Subnetting Needed?

Suppose a company has 500 computers on one network.

Problems:

- Too much network traffic

- Slow communication

- Difficult to manage

- Lower security

To solve these problems, the network is divided into smaller networks called subnets.

Example

A company has three departments:

- HR

- Accounts

- Sales

Instead of keeping all computers in one network, each department gets its own subnet.

Company Network

│

-------------------------

│                 │                   │

HR        Accounts      Sales

Subnet   Subnet      Subnet

Advantages of Subnetting

- Reduces network traffic

- Improves network speed

- Better security

- Easy network management

- Efficient use of IP addresses

Disadvantages

- Requires proper planning

- Configuration is more complex

- Network administrator must calculate subnet addresses correctly

2. Subnet Addressing

Definition

Subnet addressing is the process of assigning IP addresses to different subnets.

Each subnet has:

- Network Address

- Host Address Range

- Broadcast Address

Example

Original Network:

192.168.1.0/24

After subnetting:

Subnet 1

192.168.1.0/26

Subnet 2

192.168.1.64/26

Subnet 3

192.168.1.128/26

Subnet 4

192.168.1.192/26

Each subnet can communicate independently.

Components of a Subnet

1. Network Address

Identifies the subnet.

Example:

192.168.1.0

2. Host Address

Used by computers.

Example:

192.168.1.10

192.168.1.20

192.168.1.30

3. Broadcast Address

Used to send data to every device in the subnet.

Example:

192.168.1.255

3. Address Masking (Subnet Mask)

A Subnet Mask separates the network part and the host part of an IP address.

Simple Definition (Exam)

A subnet mask identifies the network portion and the host portion of an IP address.

Example

IP Address

192.168.1.25

Subnet Mask

255.255.255.0

Diagram

IP Address

192.168.1.25

Subnet Mask

255.255.255.0

Network Portion

192.168.1

Host Portion

25

Common Subnet Masks

| Prefix | Subnet Mask |
| --- | --- |
| /8 | 255.0.0.0 |
| /16 | 255.255.0.0 |
| /24 | 255.255.255.0 |
| /25 | 255.255.255.128 |
| /26 | 255.255.255.192 |
| /27 | 255.255.255.224 |

Advantages

- Separates network and host IDs

- Reduces unnecessary traffic

- Improves security

- Helps routers identify networks

4. Supernetting

Definition

Supernetting is the process of combining multiple smaller networks into one larger network.

Simple Definition (Exam)

Supernetting combines several small networks into a single larger network.

Why Supernetting is Needed

When routers maintain many network entries, the routing table becomes large.

Supernetting reduces the size of routing tables.

Example

Instead of keeping:

192.168.0.0

192.168.1.0

192.168.2.0

192.168.3.0

They can be combined into one larger network.

Advantages

- Smaller routing table

- Faster routing

- Better performance

- Easier network management

Difference between Subnetting and Supernetting

| Subnetting | Supernetting |
| --- | --- |
| Divides one large network | Combines many small networks |
| Creates smaller networks | Creates a larger network |
| Improves local management | Reduces routing table size |
| Used inside organizations | Mostly used by ISPs |

5. ARP (Address Resolution Protocol)

Definition

ARP converts an IP address (logical address) into a MAC address (physical address).

Simple Definition (Exam)

ARP is used to find the MAC address of a device using its IP address.

Why ARP is Needed

Computers communicate using MAC addresses on a local network.

Suppose Computer A knows Computer B's IP address but not its MAC address.

ARP helps find the MAC address.

Working of ARP

Step 1

Computer A wants to send data.

It knows:

IP = 192.168.1.20

But it does not know the MAC address.

Step 2

Computer A sends an ARP Request.

Who has IP 192.168.1.20?

Step 3

All computers receive the request.

Only Computer B replies.

Step 4

Computer B sends an ARP Reply.

My MAC Address is

AA-BB-CC-DD-EE-FF

Step 5

Computer A stores the MAC address in its ARP Cache and sends the data.

ARP Diagram

Computer A

IP : 192.168.1.10

│

ARP Request

"Who has 192.168.1.20?"

│

-----------------------------

│

Computer B

IP : 192.168.1.20

ARP Reply

MAC:

AA-BB-CC-DD-EE-FF

Advantages of ARP

- Fast communication

- Automatic MAC address discovery

- Reduces manual configuration

- Supports LAN communication

6. RARP (Reverse Address Resolution Protocol)

Definition

RARP performs the opposite operation of ARP.

It converts a MAC address into an IP address.

Simple Definition (Exam)

RARP is used to find the IP address from a MAC address.

Why RARP was Needed

Earlier, diskless computers knew only their MAC address.

They contacted a RARP Server to obtain their IP address before communicating on the network.

Working of RARP

Step 1

Computer knows only:

MAC Address

Step 2

Computer sends a RARP request.

Step 3

RARP Server receives the request.

Step 4

Server sends back the IP address.

RARP Diagram

Computer

(MAC Address)

│

RARP Request

│

RARP Server

│

Returns

IP Address

Advantages

- Automatically assigns IP addresses

- Useful for diskless systems

- Simplifies initial network configuration

Limitation

RARP has largely been replaced by BOOTP and DHCP, which provide more features such as automatic IP assignment and additional network configuration information.

Difference between ARP and RARP

| ARP | RARP |
| --- | --- |
| Finds MAC address | Finds IP address |
| IP → MAC | MAC → IP |
| Used by all computers on a LAN | Previously used by diskless computers |
| Does not require a dedicated server | Requires a RARP server |

One-Mark Questions

- Define subnet.

- What is subnet addressing?

- Define subnet mask.

- What is supernetting?

- What is ARP?

- What is RARP?

- Expand ARP.

- Expand RARP.

Two/Four-Mark Questions

- Explain subnetting with an example.

- Explain subnet addressing.

- Explain the subnet mask.

- Explain the working of ARP with a diagram.

- Explain the working of RARP with a diagram.

- Differentiate between subnetting and supernetting.

- Differentiate between ARP and RARP.

Viva Questions

- Why is subnetting used?

- What is the purpose of a subnet mask?

- What is the difference between a logical address and a physical address?

- Why is ARP required before communication on a LAN?

- Why has RARP been replaced by DHCP?

Unit 1 Quick Revision

- Subnet → Divides one large network into smaller networks.

- Subnet Addressing → Assigns IP addresses to different subnets.

- Subnet Mask → Separates the network ID and host ID.

- Supernetting → Combines multiple smaller networks into one larger network.

- ARP → Converts IP Address → MAC Address.

- RARP → Converts MAC Address → IP Address.

Unit 1 Summary

1. Internet

- A global network that connects millions of computers worldwide.

- Uses the TCP/IP protocol for communication.

- Provides services such as web browsing, email, online banking, and cloud computing.

Key Point:
Internet = Public Network

2. Intranet

- A private network used within an organization.

- Accessible only by authorized users.

- Used for sharing documents, employee information, and internal communication.

Key Point:
Intranet = Private Network

3. ISP (Internet Service Provider)

An ISP is a company that provides Internet access to users.

Functions

- Provides Internet connection

- Assigns IP addresses

- Offers email and web services

- Provides technical support

Examples

- JioFiber

- Airtel Xstream Fiber

- BSNL

- ACT Fibernet

4. ICANN (Internet Corporation for Assigned Names and Numbers)

ICANN manages Internet resources globally.

Functions

- Allocates IP addresses

- Manages domain names

- Coordinates DNS root servers

- Ensures domain names are unique

Example:

- www.google.com

- www.msbte.org.in

5. IPv4

IPv4 is the 4th version of the Internet Protocol.

Features

- 32-bit address

- 4 octets

- Decimal notation

- About 4.3 billion addresses

Example:

192.168.1.10

Advantages

- Easy to use

- Widely supported

Disadvantages

- Limited address space

- Requires NAT

6. IPv4 Header

Important fields:

- Version

- Header Length

- Type of Service

- Total Length

- Identification

- Flags

- Fragment Offset

- TTL

- Protocol

- Header Checksum

- Source Address

- Destination Address

Important Fact:
Minimum IPv4 Header Size = 20 Bytes

7. IPv6

IPv6 is the latest version of the Internet Protocol.

Features

- 128-bit address

- Hexadecimal notation

- Very large address space

- Better security

- Faster routing

- No NAT required

Example:

2001:db8:85a3::8a2e:370:7334

8. IPv6 Header

Important fields:

- Version

- Traffic Class

- Flow Label

- Payload Length

- Next Header

- Hop Limit

- Source Address

- Destination Address

Important Fact:
IPv6 Header Size = 40 Bytes (Fixed)

9. Difference Between IPv4 and IPv6

| IPv4 | IPv6 |
| --- | --- |
| 32-bit | 128-bit |
| Decimal | Hexadecimal |
| 4 octets | 8 groups |
| 4.3 billion addresses | Huge address space |
| TTL | Hop Limit |
| Uses NAT | No NAT required |

10. Subnet

A Subnet is a smaller network created by dividing a large network.

Advantages

- Reduces traffic

- Improves performance

- Better security

- Easy management

11. Subnet Addressing

Subnet addressing divides a network into multiple subnets.

Each subnet has:

- Network Address

- Host Addresses

- Broadcast Address

12. Subnet Mask (Address Masking)

A subnet mask separates the Network ID and Host ID.

Example:

IP Address   : 192.168.1.25

Subnet Mask  : 255.255.255.0

Common subnet masks:

- /8 → 255.0.0.0

- /16 → 255.255.0.0

- /24 → 255.255.255.0

13. Supernetting

Supernetting combines multiple smaller networks into one larger network.

Advantages

- Smaller routing tables

- Faster routing

- Better network management

14. ARP (Address Resolution Protocol)

ARP converts:

IP Address → MAC Address

Working

- Device knows the destination IP.

- Sends an ARP Request.

- Destination replies with its MAC address.

- Data transmission begins.

15. RARP (Reverse Address Resolution Protocol)

RARP converts:

MAC Address → IP Address

Working

- Device knows only its MAC address.

- Sends a RARP request.

- RARP server returns the IP address.

Important Differences

Internet vs Intranet

| Internet | Intranet |
| --- | --- |
| Public | Private |
| Global | Organization only |
| Less secure | More secure |

ARP vs RARP

| ARP | RARP |
| --- | --- |
| IP → MAC | MAC → IP |
| Finds MAC address | Finds IP address |

Subnetting vs Supernetting

| Subnetting | Supernetting |
| --- | --- |
| Divides one network | Combines multiple networks |
| Creates smaller networks | Creates a larger network |

Formula/Memory Points

- IPv4 = 32 bits

- IPv6 = 128 bits

- IPv4 Header = 20 bytes (minimum)

- IPv6 Header = 40 bytes (fixed)

- ARP = IP → MAC

- RARP = MAC → IP

- Subnetting = Divide

- Supernetting = Combine

Most Important MSBTE Exam Questions

2 Marks

- Define Internet.

- Define Intranet.

- What is ISP?

- What is ICANN?

- Define IPv4 and IPv6.

- Define Subnet.

- Define ARP and RARP.

4 Marks

- Compare IPv4 and IPv6.

- Explain the functions of ISP.

- Explain the functions of ICANN.

- Explain subnetting.

- Explain ARP with a diagram.

6 Marks

- Explain the IPv4 header format.

- Explain the IPv6 header format.

- Explain subnetting and supernetting with suitable examples.

- Explain the working of ARP and RARP with diagrams.