Unit 4 : Application Layer Protocols

Part 1

This part covers:

- Domain Name System (DNS)

- Electronic Mail (E-mail)

- File Transfer Protocol (FTP)

- Hypertext Transfer Protocol (HTTP)

- Hypertext Transfer Protocol Secure (HTTPS)

Introduction to Application Layer

The Application Layer is the 7th and topmost layer of the OSI Model.

It provides network services directly to the user and allows applications to communicate over a network.

Examples of Applications

- Web Browser

- Gmail

- WhatsApp

- File Transfer

- Online Banking

Main Functions

- Web browsing

- File transfer

- Email communication

- Name resolution

- Remote login

1. Domain Name System (DNS)

Definition

DNS (Domain Name System) is a system that converts a domain name into an IP address.

Simple Definition (Exam)

DNS converts human-readable domain names into IP addresses.

Why is DNS Needed?

Humans remember names more easily than numbers.

Instead of remembering:

142.250.183.78

we simply type:

www.google.com

DNS finds the corresponding IP address.

Working of DNS

Step 1

User enters:

www.google.com

Step 2

The DNS server searches for the IP address.

Step 3

The DNS server returns the IP address.

Step 4

The browser connects to Google's server.

Diagram

User

│

www.google.com

│

DNS Server

│

Returns IP Address

│

Google Server

Advantages

- Easy to remember website names

- Faster browsing

- Eliminates the need to remember IP addresses

2. Electronic Mail (E-mail)

Definition

E-mail is an application that allows users to send and receive electronic messages over the Internet.

Simple Definition (Exam)

E-mail is a service used to exchange electronic messages over a network.

Components of an Email

- Sender Address

- Receiver Address

- Subject

- Message Body

- Attachments

Working of E-mail

- User writes an email.

- Email is sent to the mail server.

- Mail server forwards it to the receiver's mail server.

- Receiver downloads and reads the email.

Email Protocols

SMTP (Simple Mail Transfer Protocol)

- Used for sending emails.

POP3 (Post Office Protocol Version 3)

- Downloads emails to the user's device.

IMAP (Internet Message Access Protocol)

- Synchronizes emails across multiple devices.

Advantages

- Fast communication

- Supports attachments

- Low cost

- Global communication

3. File Transfer Protocol (FTP)

Definition

FTP (File Transfer Protocol) is a protocol used to transfer files between computers over a network.

Simple Definition (Exam)

FTP is used to upload and download files over a network.

Uses of FTP

- Upload website files

- Download software

- Transfer documents

- Share large files

Working of FTP

- Client connects to FTP server.

- User logs in.

- Files are uploaded or downloaded.

- Connection is closed.

Diagram

Client

│

FTP Request

│

FTP Server

│

Upload / Download Files

Advantages

- Fast file transfer

- Supports large files

- Easy file sharing

Disadvantages

- Data is not encrypted

- Less secure than modern protocols

4. Hypertext Transfer Protocol (HTTP)

Definition

HTTP (Hypertext Transfer Protocol) is the protocol used for communication between a web browser and a web server.

Simple Definition (Exam)

HTTP is the protocol used to transfer web pages over the Internet.

Working

- Browser sends an HTTP request.

- Web server receives the request.

- Server sends the requested webpage.

- Browser displays the webpage.

Diagram

Browser

│

HTTP Request

│

Web Server

│

HTTP Response

│

Browser

Features

- Simple protocol

- Fast communication

- Stateless protocol

- Used for web browsing

Advantages

- Easy to implement

- Supports web communication

- Widely used

Disadvantages

- No encryption

- Less secure

- Data can be intercepted

5. Hypertext Transfer Protocol Secure (HTTPS)

Definition

HTTPS (Hypertext Transfer Protocol Secure) is the secure version of HTTP that uses TLS/SSL encryption.

Simple Definition (Exam)

HTTPS is a secure version of HTTP that encrypts data between the browser and the web server.

Features

- Secure communication

- Data encryption

- Authentication

- Data integrity

Working

- Browser requests a secure connection.

- Server sends its digital certificate.

- A secure encrypted connection is established.

- Data is exchanged securely.

Diagram

Browser

│

HTTPS Request

│

Encrypted Connection

│

Web Server

Advantages

- Secure communication

- Protects passwords

- Protects banking transactions

- Prevents data theft

Applications

- Online Banking

- E-commerce

- Government websites

- Login pages

Difference Between HTTP and HTTPS

| HTTP | HTTPS |
| --- | --- |
| Not secure | Secure |
| No encryption | Uses TLS/SSL encryption |
| Port 80 | Port 443 |
| Used for general websites | Used for secure websites |
| Less secure | More secure |

Memory Tricks

DNS

Name → IP Address

SMTP

Send Mail

POP3

Download Mail

IMAP

Synchronize Mail

FTP

File Transfer

HTTP

Web Pages (Not Secure)

HTTPS

Secure Web Pages

One-Mark Questions

- Define DNS.

- Define E-mail.

- What is FTP?

- Define HTTP.

- Define HTTPS.

- Expand DNS.

- Expand FTP.

- Expand HTTP.

- Expand HTTPS.

- What is SMTP?

Two/Four-Mark Questions

- Explain DNS.

- Explain the working of E-mail.

- Explain FTP.

- Explain HTTP.

- Explain HTTPS.

- Differentiate between HTTP and HTTPS.

- Differentiate between POP3 and IMAP.

Viva Questions

- What is the purpose of DNS?

- Which protocol is used to send emails?

- Which protocol is used to transfer files?

- Which protocol is used for secure web browsing?

- What is the difference between HTTP and HTTPS?

- Which port is used by HTTP and HTTPS?

Quick Revision

- Application Layer → Provides services directly to users.

- DNS → Converts Domain Name → IP Address.

- E-mail → Electronic message service.

- SMTP → Sends emails.

- POP3 → Downloads emails.

- IMAP → Synchronizes emails.

- FTP → Transfers files between computers.

- HTTP → Transfers web pages (Port 80).

- HTTPS → Secure version of HTTP using TLS/SSL (Port 443).

Part 2

This part covers:

- Dynamic Host Configuration Protocol (DHCP)

- DHCP Version 6 (DHCPv6)

- Simple Network Management Protocol (SNMP)

- Network Management

- Remote Login (TELNET & SSH)

1. Dynamic Host Configuration Protocol (DHCP)

Definition

DHCP (Dynamic Host Configuration Protocol) is a protocol that automatically assigns IP addresses and other network settings to devices connected to a network.

Simple Definition (Exam)

DHCP automatically assigns IP addresses to devices on a network.

Why is DHCP Needed?

Without DHCP, the network administrator must manually assign an IP address to every device.

With DHCP, devices automatically receive:

- IP Address

- Subnet Mask

- Default Gateway

- DNS Server Address

Working of DHCP (DORA Process)

DHCP works in 4 steps, known as the DORA Process.

1. Discover (D)

The client searches for a DHCP server.

2. Offer (O)

The DHCP server offers an available IP address.

3. Request (R)

The client requests the offered IP address.

4. Acknowledge (A)

The DHCP server confirms and assigns the IP address.

Diagram

Client                     DHCP Server

Discover ------------------>

<---------------- Offer

Request ------------------->

<---------------- Acknowledge

Advantages

- Automatic IP assignment

- Reduces configuration errors

- Saves time

- Easy network management

Disadvantages

- If the DHCP server fails, new devices may not receive IP addresses.

- Requires a DHCP server.

2. DHCPv6 (Dynamic Host Configuration Protocol Version 6)

Definition

DHCPv6 is the version of DHCP used with IPv6 networks.

Simple Definition (Exam)

DHCPv6 automatically assigns IPv6 addresses and network information.

Features

- Supports IPv6

- Automatic IPv6 address assignment

- Provides DNS and gateway information

- Suitable for modern networks

Difference Between DHCP and DHCPv6

| DHCP | DHCPv6 |
| --- | --- |
| Works with IPv4 | Works with IPv6 |
| Assigns IPv4 addresses | Assigns IPv6 addresses |
| Uses IPv4 networks | Uses IPv6 networks |

3. Simple Network Management Protocol (SNMP)

Definition

SNMP (Simple Network Management Protocol) is a protocol used to monitor and manage network devices.

Simple Definition (Exam)

SNMP is used to monitor and manage network devices.

Devices Managed by SNMP

- Routers

- Switches

- Servers

- Printers

- Firewalls

Components of SNMP

1. SNMP Manager

- Monitors network devices.

- Sends requests to agents.

2. SNMP Agent

- Software running on the network device.

- Collects device information.

3. MIB (Management Information Base)

- A database containing information about network devices.

Diagram

SNMP Manager

│

SNMP Request

│

SNMP Agent

│

Returns Device Information

Advantages

- Easy network monitoring

- Detects network faults

- Centralized management

- Improves network performance

4. Network Management

Definition

Network Management is the process of monitoring, controlling, maintaining, and securing a computer network.

Simple Definition (Exam)

Network management is the process of managing and maintaining a network efficiently.

Objectives

- Improve performance

- Detect faults

- Increase security

- Maintain availability

- Reduce downtime

FCAPS Model

Network management is divided into five areas, called FCAPS.

F – Fault Management

Detects and fixes network problems.

Example:

- Router failure

- Cable damage

C – Configuration Management

Manages network settings and configurations.

Example:

- IP configuration

- Router configuration

A – Accounting Management

Tracks network usage.

Example:

- Internet usage reports

- User activity

P – Performance Management

Measures network performance.

Example:

- Bandwidth

- Speed

- Traffic

S – Security Management

Protects the network from unauthorized access.

Example:

- Firewalls

- Passwords

- Encryption

Advantages

- Better network performance

- Improved security

- Faster troubleshooting

- Reduced downtime

5. Remote Login

Definition

Remote Login allows a user to access and control another computer over a network.

Simple Definition (Exam)

Remote login allows users to access another computer from a remote location.

TELNET

Definition

TELNET is a remote login protocol that allows users to access another computer over a network.

Features

- Text-based communication

- No encryption

- Less secure

- Uses Port 23

Advantages

- Easy to use

- Simple configuration

Disadvantages

- Passwords are sent in plain text.

- Not secure.

SSH (Secure Shell)

Definition

SSH (Secure Shell) is a secure remote login protocol that encrypts all communication.

Simple Definition (Exam)

SSH is a secure protocol used for remote login.

Features

- Encrypted communication

- Secure authentication

- Secure file transfer support

- Uses Port 22

Advantages

- Secure communication

- Protects passwords

- Prevents hacking

- Widely used

Difference Between TELNET and SSH

| TELNET | SSH |
| --- | --- |
| Not secure | Secure |
| No encryption | Uses encryption |
| Port 23 | Port 22 |
| Password sent in plain text | Password encrypted |
| Less secure | More secure |

Memory Tricks

DHCP

Automatically assigns IP Address

DHCPv6

DHCP for IPv6

SNMP

Monitor Network Devices

FCAPS

F – Fault

C – Configuration

A – Accounting

P – Performance

S – Security

TELNET

Text + No Security

Port 23

SSH

Secure Shell

Port 22

One-Mark Questions

- Define DHCP.

- Define DHCPv6.

- What is SNMP?

- Define Network Management.

- What is TELNET?

- What is SSH?

- Expand DHCP.

- Expand SNMP.

- Expand SSH.

- What is FCAPS?

Two/Four-Mark Questions

- Explain the working of DHCP.

- Explain DHCPv6.

- Explain SNMP with components.

- Explain Network Management.

- Explain FCAPS.

- Explain TELNET.

- Explain SSH.

- Differentiate between TELNET and SSH.

- Differentiate between DHCP and DHCPv6.

Viva Questions

- What is the purpose of DHCP?

- What are the four steps of the DHCP process?

- Which protocol monitors network devices?

- What does FCAPS stand for?

- Which remote login protocol is more secure?

- What are the port numbers of TELNET and SSH?

Quick Revision

- DHCP → Automatically assigns IPv4 addresses (DORA: Discover → Offer → Request → Acknowledge).

- DHCPv6 → Automatically assigns IPv6 addresses.

- SNMP → Monitors and manages network devices.

- Network Management → Uses the FCAPS model:

  - F – Fault

  - C – Configuration

  - A – Accounting

  - P – Performance

  - S – Security

- TELNET → Remote login, Port 23, not secure.

- SSH → Secure remote login, Port 22, encrypted.

Unit 4 Summary

1. Application Layer

The Application Layer is the 7th (topmost) layer of the OSI Model.

Main Functions

- Web browsing

- Email communication

- File transfer

- Name resolution

- Remote login

- Network management

Examples

- Google Chrome

- Gmail

- WhatsApp

- FileZilla

- Online Banking

Key Point:
Application Layer = Provides services directly to users.

2. Domain Name System (DNS)

DNS (Domain Name System) converts a domain name into an IP address.

Example

- www.google.com → 142.250.xxx.xxx

Advantages

- Easy to remember website names

- Faster access to websites

- No need to remember IP addresses

Key Point:
DNS = Domain Name → IP Address

3. Electronic Mail (E-mail)

E-mail is used to send and receive electronic messages over the Internet.

Email Protocols

- SMTP → Sends emails

- POP3 → Downloads emails

- IMAP → Synchronizes emails across multiple devices

Components of an Email

- Sender

- Receiver

- Subject

- Message

- Attachment

Key Point:

- SMTP = Send

- POP3 = Download

- IMAP = Synchronize

4. File Transfer Protocol (FTP)

FTP (File Transfer Protocol) is used to transfer files between computers.

Uses

- Upload website files

- Download software

- Share documents

Advantages

- Fast file transfer

- Supports large files

Disadvantage

- No encryption (less secure)

Key Point:
FTP = File Upload & Download

5. Hypertext Transfer Protocol (HTTP)

HTTP is the protocol used for communication between a web browser and a web server.

Features

- Transfers web pages

- Fast

- Stateless

- Port 80

Disadvantage

- No encryption

- Less secure

Key Point:
HTTP = Web Pages (Not Secure)

6. Hypertext Transfer Protocol Secure (HTTPS)

HTTPS is the secure version of HTTP that uses TLS/SSL encryption.

Features

- Secure communication

- Data encryption

- Authentication

- Data integrity

- Port 443

Applications

- Online Banking

- E-commerce

- Login pages

Key Point:
HTTPS = Secure Web Browsing

Difference Between HTTP and HTTPS

| HTTP | HTTPS |
| --- | --- |
| Not secure | Secure |
| No encryption | Uses TLS/SSL |
| Port 80 | Port 443 |
| Used for normal websites | Used for secure websites |

7. Dynamic Host Configuration Protocol (DHCP)

DHCP automatically assigns IPv4 addresses and other network settings to devices.

DORA Process

- Discover

- Offer

- Request

- Acknowledge

Advantages

- Automatic IP assignment

- Saves time

- Reduces configuration errors

Key Point:
DHCP = Automatic IPv4 Address Assignment

8. DHCPv6

DHCPv6 automatically assigns IPv6 addresses.

Difference

| DHCP | DHCPv6 |
| --- | --- |
| IPv4 | IPv6 |
| Assigns IPv4 addresses | Assigns IPv6 addresses |

Key Point:
DHCPv6 = DHCP for IPv6

9. Simple Network Management Protocol (SNMP)

SNMP is used to monitor and manage network devices.

Components

- SNMP Manager

- SNMP Agent

- MIB (Management Information Base)

Managed Devices

- Routers

- Switches

- Servers

- Printers

Key Point:
SNMP = Monitor Network Devices

10. Network Management

Network Management is the process of monitoring, maintaining, and securing a computer network.

FCAPS Model

- F – Fault Management

- C – Configuration Management

- A – Accounting Management

- P – Performance Management

- S – Security Management

Key Point:
FCAPS = Fault, Configuration, Accounting, Performance, Security

11. Remote Login

Remote Login allows users to access another computer through a network.

TELNET

Features

- Text-based

- No encryption

- Port 23

- Less secure

Key Point : TELNET = Remote Login (Not Secure)

SSH (Secure Shell)

Features

- Encrypted communication

- Secure authentication

- Port 22

Advantages

- Secure

- Protects passwords

- Widely used

Key Point : SSH = Secure Remote Login

Difference Between TELNET and SSH

| TELNET | SSH |
| --- | --- |
| Not secure | Secure |
| No encryption | Uses encryption |
| Port 23 | Port 22 |
| Password sent in plain text | Password encrypted |

Memory Tricks

- DNS → Name → IP Address

- SMTP → Send Mail

- POP3 → Download Mail

- IMAP → Synchronize Mail

- FTP → File Transfer

- HTTP → Web Pages (Port 80)

- HTTPS → Secure Web Pages (Port 443)

- DHCP → Automatic IPv4 Address

- DHCPv6 → Automatic IPv6 Address

- SNMP → Monitor Devices

- FCAPS → Fault, Configuration, Accounting, Performance, Security

- TELNET → Port 23 (Not Secure)

- SSH → Port 22 (Secure)

Most Important MSBTE Exam Questions

2 Marks

- Define DNS.

- Define FTP.

- Define HTTP.

- Define HTTPS.

- Define DHCP.

- Define SNMP.

- Define TELNET.

- Define SSH.

4 Marks

- Explain DNS.

- Explain the working of E-mail.

- Explain FTP.

- Explain DHCP using the DORA process.

- Explain SNMP with its components.

- Explain FCAPS.

6 Marks

- Differentiate between HTTP and HTTPS.

- Differentiate between TELNET and SSH.

- Explain Network Management with the FCAPS model.

- Explain DHCP with a diagram.

- Explain SSH with its features.

Quick Revision

- Application Layer → Provides network services directly to users.

- DNS → Converts Domain Name → IP Address.

- E-mail → Uses SMTP, POP3, and IMAP.

- FTP → Transfers files between computers.

- HTTP → Transfers web pages (Port 80).

- HTTPS → Secure web communication using TLS/SSL (Port 443).

- DHCP → Automatically assigns IPv4 addresses using DORA (Discover → Offer → Request → Acknowledge).

- DHCPv6 → Automatically assigns IPv6 addresses.

- SNMP → Monitors and manages network devices.

- FCAPS → Fault, Configuration, Accounting, Performance, Security.

- TELNET → Remote login, Port 23, not secure.

- SSH → Secure remote login, Port 22, encrypted.