# Calculator using Assembly Language

Department of Computer Engineering

Group Members : -

1. Danish Solkar (24111590805)

2. Kalam Khan (24111590782)

3. Jareer Khan (24111590791)

4. Saud Kazi (24111590790)

| # Sr. no. | # INDEX | Page no. |
| --- | --- | --- |
| 1 | Introduction | 3 |
| 2 | Algorithm | 3 – 5 |
| 3 | Flowchart | 6 |
| 4 | Program | 7 – 11 |
| 5 | Output Of Program | 12 |
| 6 | Application | 13 |
| 7 | Conclusion | 13 |
| 8 | References | 13 |

# 1. INTRODUCTION

A calculator is one of the most basic yet important computational tools used in digital systems. Implementing a calculator using assembly language provides a deep understanding of how arithmetic operations are executed at the processor level.

This project focuses on creating a simple calculator program using the 8086-microprocessor instruction set. Unlike high-level programming languages, assembly language requires direct interaction with registers, memory, and system interrupts. Therefore, this project demonstrates how user input is captured character by character, converted into numeric form, processed, and then converted back into displayable output.

The calculator is menu-driven, meaning the user selects an arithmetic operator, and the program performs the corresponding operation. This makes the program interactive and structured.

# 2. Algorithm

1) Start the program

2) Initialize data segment

3) Accept the first number

- Display message: “Enter First Number”.

- Initialize a variable (accumulator) to store the number (initial value = 0).

- Read characters from keyboard one by one.

- For each character:

  - If Enter key (carriage return) is pressed → stop reading digits.

  - Otherwise:

    - Convert ASCII digit into numeric value (subtract 48).

    - Multiply current number by 10.

    - Add new digit to it.

- Store the complete multi-digit value as NUM1.

4) Accept the operator

- Display message: “Enter Operator (+, −, *, /)”.

- Read one character from keyboard.

- Store this character as the selected operator.

5) Accept the second number

- Display message: “Enter Second Number”.

- Repeat the same procedure used for the first number:

  - Read digits one by one.

  - Convert ASCII to numeric.

  - Multiply previous value by 10.

  - Add new digit.

- Store the final value as NUM2.

6) Decide which operation to perform

Compare the entered operator with valid operators:

- If operator is ‘+’

  - Load first number into AX.

  - Add second number.

  - Store result.

- Else if operator is ‘−’

  - Load first number into AX.

  - Subtract second number.

  - Store result.

- Else if operator is ‘*’

  - Load first number into AX.

  - Multiply by second number.

  - Store lower 16-bit result.

- Else if operator is ‘/’

  - Clear DX (required before division).

  - Load first number into AX.

  - Divide by second number.

  - Store quotient as result.

- Else (operator is not valid)

  - Display “Invalid” message.

  - Terminate program.

7) Display the result

- Display message: “Result:”.

- Take the numeric result and convert it into digits:

  - Divide number repeatedly by 10.

  - Store remainders (digits) on stack.

- Pop digits from stack one by one.

- Convert each digit to ASCII (add 48).

- Display digits in correct order.

8) Terminate the program

- Return control to DOS using interrupt.

9) End program

# 3. Flowchart

# 4. PROGRAM

.MODEL SMALL

.STACK 100H

.DATA

MSG1 DB 10,13,'ENTER FIRST NUMBER: $'

MSG2 DB 10,13,'ENTER OPERATOR (+, -, *, /): $'

MSG3 DB 10,13,'ENTER SECOND NUMBER: $'

MSG4 DB 10,13,'RESULT: $'

ERR_MSG DB 10,13,'INVALID! $'

NUM1 DW ?

NUM2 DW ?

; We use two words to store a 32-bit result

RES_LOW  DW 0

RES_HIGH DW 0

.CODE

START:

MOV AX, @DATA

MOV DS, AX

; Get First Number

LEA DX, MSG1

MOV AH, 09H

INT 21H

CALL READ_NUM

MOV NUM1, AX

; Get Operator

LEA DX, MSG2

MOV AH, 09H

INT 21H

MOV AH, 01H

INT 21H

MOV BL, AL

; Get Second Number

LEA DX, MSG3

MOV AH, 09H

INT 21H

CALL READ_NUM

MOV NUM2, AX

; Decision Logic

CMP BL, '+'

JE  DO_ADD

CMP BL, '-'

JE  DO_SUB

CMP BL, '*'

JE  DO_MUL

CMP BL, '/'

JE  DO_DIV

JMP ERROR_EXIT

DO_ADD:

MOV AX, NUM1

ADD AX, NUM2

MOV RES_LOW, AX

MOV RES_HIGH, 0    ; No overflow possible in 16-bit add for small nums

JMP SHOW_RESULT

DO_SUB:

MOV AX, NUM1

SUB AX, NUM2

MOV RES_LOW, AX

MOV RES_HIGH, 0

JMP SHOW_RESULT

DO_MUL:

MOV AX, NUM1

MUL NUM2           ; DX:AX = AX * NUM2

MOV RES_LOW, AX    ; Lower 16 bits

MOV RES_HIGH, DX   ; Upper 16 bits (Fixes the 9578 error)

JMP SHOW_RESULT

DO_DIV:

MOV DX, 0

MOV AX, NUM1

DIV NUM2

MOV RES_LOW, AX

MOV RES_HIGH, 0

JMP SHOW_RESULT

ERROR_EXIT:

LEA DX, ERR_MSG

MOV AH, 09H

INT 21H

JMP FINISH

SHOW_RESULT:

LEA DX, MSG4

MOV AH, 09H

INT 21H

CALL WRITE_NUM_32  ; Call the updated 32-bit print routine

FINISH:

MOV AH, 4CH

INT 21H

; --- PROCEDURE: READ MULTI-DIGIT NUMBER ---

READ_NUM PROC

PUSH BX

PUSH CX

MOV BX, 0

READ_LOOP:

MOV AH, 01H

INT 21H

CMP AL, 13

JE READ_DONE

SUB AL, 48

MOV CL, AL

MOV CH, 0

MOV AX, 10

MUL BX

ADD AX, CX

MOV BX, AX

JMP READ_LOOP

READ_DONE:

MOV AX, BX

POP CX

POP BX

RET

READ_NUM ENDP

; --- PROCEDURE: WRITE 32-BIT NUMBER (DX:AX) ---

WRITE_NUM_32 PROC

PUSH AX

PUSH BX

PUSH CX

PUSH DX

MOV AX, RES_LOW

MOV DX, RES_HIGH

MOV BX, 10

MOV CX, 0

CONVERT_LOOP:

; Standard 32-bit division logic for 8086

; To avoid overflow, we divide the high part, then the low part

PUSH AX            ; Save low part

MOV AX, DX         ; Move high part to AX

MOV DX, 0          ; Clear DX

DIV BX             ; DX:AX / 10 -> AX = High Quotient, DX = High Remainder

MOV BP, AX         ; Store High Quotient in BP

POP AX             ; Restore low part

DIV BX             ; DX:AX / 10 -> AX = Low Quotient, DX = Real Remainder

PUSH DX            ; Save the actual digit

INC CX

MOV DX, BP         ; Put High Quotient back into DX for next loop

OR AX, DX          ; Check if both DX and AX are 0

JNZ CONVERT_LOOP

DISPLAY_LOOP:

POP DX

ADD DL, 48

MOV AH, 02H

INT 21H

LOOP DISPLAY_LOOP

POP DX

POP CX

POP BX

POP AX

RET

WRITE_NUM_32 ENDP

END START

# 5. OUTPUT OF THE PROGRAM

# 6. APPLICATIONS

1. Educational Learning Tool
Helps students understand low-level programming, microprocessor operations, and assembly language structure.

2. Understanding Number Conversion Techniques
Demonstrates conversion between ASCII and numeric formats.

3. Foundation for Embedded Systems Programming
Many embedded systems require direct hardware control similar to assembly programming.

4. Debugging and Performance Optimization
Learning assembly helps programmers understand how high-level programs are executed internally.

5. Basic Arithmetic Processing in Low-Memory Systems
Useful where minimal resources are available.

# 7. CONCLUSION

The menu-driven calculator implemented using 8086 assembly language successfully performs basic arithmetic operations on multi-digit numbers. The project demonstrates essential microprocessor programming concepts such as register manipulation, interrupt handling, procedure calls, control flow, and data conversion. It highlights how simple operations in high-level languages require multiple low-level steps when implemented directly on hardware.

Overall, this project strengthens understanding of processor-level computation and structured assembly programming.

# 8. REFERENCES

- Douglas V. Hall — Microprocessors and Interfacing: Programming and Hardware.

- Kip R. Irvine — Assembly Language for x86 Processors.

- Intel 8086 Microprocessor Programmer’s Reference Manual by Intel.

- Ramesh S. Gaonkar — Microprocessor Architecture, Programming, and Applications.