Practical No. 11

Aim : Write an ALP to concatenate two strings

Name : Danish Altaf Solkar 						Class : CO - B

Enrolment no. : 24111590805 						Batch : 2

Code :-

assume cs:code

assume ds:data

data segment

str1 DB "Danish$"

str2 DB "Solkar$"

msg1 DB 10,13,"Str1 before concatenation : $"

msg2 DB 10,13,"Str2 after concatenation : $"

data ends

code segment

start:

MOV AX,data

MOV DS,AX

MOV AX,0000H

LEA SI,str1

LEA DI,str2

LEA DX,msg1

MOV AH,09H

INT 21H

LEA DX,str1

MOV AH,09H

INT 21H

UP:

INC SI

MOV AL,[SI]

CMP AL,'$'

JNE UP

UP1:

MOV AL,[DI]

MOV [SI],AL

INC SI

INC DI

CMP AL,'$'

JNE UP1

LEA DX,msg2

MOV AH,09H

INT 21H

LEA DX,str1

MOV AH,09H

INT 21H

MOV AH,4CH

INT 21H

code ends

end start

OUTPUT :-

| Process related (15) | Product related (10) | Total Marks (25) | Signature |
| --- | --- | --- | --- |