Unit 2 : Routing Protocols

Part 1

This part covers:

- Router Architecture

- Routing Table

- Queueing

- Switching

- Introduction to Routing Protocols

1. Router

Definition

A Router is a networking device that connects two or more different networks and forwards data packets from the source network to the destination network.

Simple Definition (Exam)

A router is a device that connects different networks and forwards data using IP addresses.

Why is a Router Needed?

Imagine two colleges have separate networks. If they want to exchange data, they need a router to connect both networks.

Without a router:

- Networks cannot communicate.

- Data cannot reach another network.

Functions of a Router

- Connects multiple networks.

- Forwards data packets.

- Selects the best path for data.

- Maintains a routing table.

- Reduces unnecessary network traffic.

Router Architecture

A router mainly consists of:

- Input Ports

- Switching Fabric

- Output Ports

- Routing Processor

Diagram

Router

+--------------------------------+

|        Routing Processor        |

+--------------------------------+

| Input | Switching | Output      |

| Ports |  Fabric   | Ports       |

+--------------------------------+

1. Input Ports

- Receive incoming packets.

- Check the destination IP address.

- Send packets to the switching fabric.

2. Switching Fabric

- Internal connection inside the router.

- Transfers packets from input ports to the correct output ports.

3. Output Ports

- Send packets to the next router or destination.

4. Routing Processor

- Controls router operations.

- Maintains the routing table.

- Runs routing protocols such as RIP, OSPF, and BGP.

Advantages of Router

- Connects different networks.

- Chooses the best route.

- Improves network efficiency.

- Reduces broadcast traffic.

- Provides better security.

2. Routing Table

Definition

A Routing Table is a database stored inside a router that contains information about network paths.

Simple Definition (Exam)

A routing table stores information about available network routes and helps the router forward packets.

Contents of a Routing Table

- Destination Network

- Subnet Mask

- Next Hop

- Interface

- Metric (Cost)

Example Routing Table

| Destination | Next Hop | Interface |
| --- | --- | --- |
| 192.168.1.0 | Direct | LAN1 |
| 192.168.2.0 | 10.0.0.2 | LAN2 |
| 172.16.0.0 | 10.0.0.3 | WAN |

Working of Routing Table

- Packet arrives at the router.

- Router reads the destination IP.

- Searches the routing table.

- Finds the best route.

- Sends the packet through the correct interface.

Advantages

- Fast routing

- Efficient packet forwarding

- Best path selection

- Reduces delays

3. Queueing

Definition

Queueing is the process of temporarily storing packets in a queue when many packets arrive at the same time.

Simple Definition (Exam)

Queueing is the temporary storage of packets before transmission.

Why Queueing is Needed?

Sometimes many packets reach the router together.

The router cannot send all packets simultaneously.

So, packets wait in a queue.

Example

Think of customers standing in a queue at a bank.

The teller serves one customer at a time.

Similarly, the router sends one packet at a time.

Diagram

Packets

↓

+----------------+

| Router Queue   |

+----------------+

↓

Packet Sent

Advantages

- Prevents packet loss

- Maintains packet order

- Reduces congestion

- Improves network performance

Disadvantages

- Increased delay

- Queue overflow may cause packet loss

4. Switching

Definition

Switching is the process of forwarding packets from one device to another through the network.

Simple Definition (Exam)

Switching is the process of transferring data from the source to the destination.

Types of Switching

1. Circuit Switching

- A dedicated communication path is established before data transfer.

- Used in traditional telephone networks.

Advantages

- Reliable

- Fixed bandwidth

Disadvantages

- Wastes bandwidth if the line is idle.

2. Packet Switching

- Data is divided into packets.

- Each packet may travel through different paths.

- Used on the Internet.

Advantages

- Efficient use of bandwidth

- Fast communication

- Reliable

3. Message Switching

- The complete message is stored and then forwarded.

- No dedicated path is required.

Disadvantages

- High delay

Difference Between Circuit and Packet Switching

| Circuit Switching | Packet Switching |
| --- | --- |
| Dedicated path | No dedicated path |
| Continuous connection | Packets travel independently |
| Used in telephone systems | Used on the Internet |
| More expensive | More efficient |

5. Routing Protocols

Definition

A Routing Protocol is a set of rules that helps routers exchange routing information and find the best path for data packets.

Simple Definition (Exam)

A routing protocol helps routers determine the best path for forwarding packets.

Why Routing Protocols are Needed?

- To find the shortest path.

- To update routing tables automatically.

- To avoid routing loops.

- To improve network performance.

Types of Routing

1. Static Routing

- Routes are entered manually by the network administrator.

- Suitable for small networks.

Advantages

- Simple

- Secure

Disadvantages

- Not suitable for large networks

- Manual updates required

2. Dynamic Routing

- Routes are updated automatically.

- Uses routing protocols.

Examples:

- RIP

- OSPF

- BGP

Advantages

- Automatic updates

- Suitable for large networks

- Adapts to network changes

Difference Between Static and Dynamic Routing

| Static Routing | Dynamic Routing |
| --- | --- |
| Manual configuration | Automatic configuration |
| Small networks | Large networks |
| No routing protocol | Uses routing protocols |
| Low maintenance for small setups | Adapts automatically to changes |

Important Points to Remember

- Router → Connects different networks.

- Routing Table → Stores path information.

- Queueing → Temporarily stores packets before transmission.

- Switching → Transfers data between devices.

- Routing Protocol → Finds the best path for packets.

One-Mark Questions

- Define router.

- Define routing table.

- What is queueing?

- What is switching?

- Define routing protocol.

- What is static routing?

- What is dynamic routing?

Two/Four-Mark Questions

- Explain router architecture with a diagram.

- Explain the routing table.

- Explain queueing.

- Explain switching.

- Differentiate between static and dynamic routing.

- Differentiate between circuit switching and packet switching.

Viva Questions

- What is the function of a router?

- What information is stored in a routing table?

- Why is queueing required?

- Which switching technique is used on the Internet?

- What is the difference between static and dynamic routing?

Quick Revision

- Router → Connects networks and forwards packets.

- Routing Table → Stores destination routes.

- Queueing → Holds packets temporarily.

- Switching → Moves data from source to destination.

- Static Routing → Manual routing.

- Dynamic Routing → Automatic routing using protocols.

Part 2

This part covers:

- Distance Vector Routing

- RIP Version 2 (RIPv2)

- Link State Routing

- OSPF (Open Shortest Path First)

- BGP (Border Gateway Protocol)

- ICMP (Internet Control Message Protocol)

1. Distance Vector Routing

Definition

Distance Vector Routing is a dynamic routing method in which each router shares its routing table with its neighboring routers to determine the best path.

Simple Definition (Exam)

Distance Vector Routing is a routing algorithm where routers exchange routing information with their neighbors.

How it Works

- Every router maintains a routing table.

- The router sends its routing table to neighboring routers at regular intervals.

- Neighboring routers compare the information.

- If a shorter path is found, the routing table is updated.

Diagram

Router A ---- Router B ---- Router C

↕                        ↕                         ↕

Routing        Routing            Routing

Table              Table                 Table

Metric Used

The distance is measured using Hop Count.

- One router crossed = 1 Hop

- Two routers crossed = 2 Hops

Example:

A → B → C

Hop Count = 2

Advantages

- Simple to configure

- Suitable for small networks

- Low processing requirements

Disadvantages

- Slow convergence

- Routing loops may occur

- Not suitable for large networks

2. RIP Version 2 (RIPv2)

Definition

RIP (Routing Information Protocol) is a Distance Vector Routing Protocol that uses hop count to select the best route.

Simple Definition (Exam)

RIP is a distance vector routing protocol that uses hop count as its routing metric.

Features of RIPv2

- Uses Hop Count

- Maximum hop count = 15

- Hop count 16 = Unreachable

- Supports subnet masks (CIDR/VLSM)

- Sends updates every 30 seconds

Working of RIPv2

- Router sends routing updates every 30 seconds.

- Neighboring routers receive the updates.

- They compare hop counts.

- The shortest route is selected.

Advantages

- Easy to configure

- Automatic route updates

- Good for small networks

Disadvantages

- Slow convergence

- Limited to 15 hops

- Not suitable for large networks

Important Exam Points

- Protocol Type: Distance Vector

- Metric: Hop Count

- Maximum Hop Count: 15

- Update Interval: 30 Seconds

3. Link State Routing

Definition

Link State Routing is a routing method in which every router has complete information about the network topology.

Simple Definition (Exam)

Link State Routing is a routing algorithm in which routers maintain a complete map of the network.

How it Works

- Each router discovers its neighbors.

- It measures the cost of each link.

- It creates a Link State Advertisement (LSA).

- LSAs are shared with all routers.

- Every router builds the same network map.

- The shortest path is calculated.

Diagram

Router A

/                      \

Router B ---- Router C

\                       /

Router D

Each router knows the complete network structure.

Advantages

- Fast convergence

- No routing loops

- Suitable for large networks

- More accurate routing

Disadvantages

- Higher memory usage

- More CPU processing

- Complex configuration

Difference Between Distance Vector and Link State Routing

| Distance Vector | Link State |
| --- | --- |
| Shares routing table with neighbors | Shares network topology information |
| Knows only neighboring routes | Knows complete network map |
| Slow convergence | Fast convergence |
| Less memory required | More memory required |
| Example: RIP | Example: OSPF |

4. OSPF (Open Shortest Path First)

Definition

OSPF is a Link State Routing Protocol used to find the shortest path between routers.

Simple Definition (Exam)

OSPF is a link state routing protocol that finds the shortest path using cost.

Features

- Link State Protocol

- Fast convergence

- Uses Cost as the routing metric

- Suitable for large networks

- Supports hierarchical routing

Working

- Routers exchange Link State Advertisements (LSAs).

- Every router creates the same network map.

- OSPF calculates the shortest path.

- Data is forwarded along the lowest-cost path.

Advantages

- Fast routing

- Reliable

- Scalable

- No routing loops

Disadvantages

- Complex configuration

- High memory usage

Important Exam Points

- Protocol Type: Link State

- Metric: Cost

- Algorithm: Dijkstra's Shortest Path Algorithm

5. BGP (Border Gateway Protocol)

Definition

BGP is a routing protocol used to exchange routing information between different Autonomous Systems (AS) on the Internet.

Simple Definition (Exam)

BGP is a routing protocol used between different Autonomous Systems on the Internet.

Autonomous System (AS)

An Autonomous System is a group of networks managed by a single organization.

Examples:

- Internet Service Providers (ISPs)

- Large companies

- Universities

Features of BGP

- Used on the Internet

- Connects different Autonomous Systems

- Highly scalable

- Reliable

- Uses path information to select routes

Advantages

- Suitable for very large networks

- Highly reliable

- Prevents routing loops

- Supports Internet-wide routing

Disadvantages

- Complex configuration

- Slow convergence compared to OSPF

- Requires more resources

Important Exam Point

BGP is mainly used by Internet Service Providers (ISPs).

6. ICMP (Internet Control Message Protocol)

Definition

ICMP is a protocol used for reporting network errors and testing network connectivity.

Simple Definition (Exam)

ICMP is used to send error messages and diagnostic information in a network.

Functions of ICMP

- Reports network errors

- Checks network connectivity

- Helps troubleshoot network problems

Uses of ICMP

1. Ping

Checks whether another computer is reachable.

Example:

ping 192.168.1.1

2. Traceroute

Shows the path taken by packets to reach the destination.

Example:

tracert www.google.com

Common ICMP Messages

- Echo Request

- Echo Reply

- Destination Unreachable

- Time Exceeded

- Redirect Message

Advantages

- Detects network problems

- Tests connectivity

- Helps troubleshoot networks

Disadvantages

- Does not transfer user data

- Can be blocked for security reasons

Comparison of Routing Protocols

| Protocol | Type | Metric | Best For |
| --- | --- | --- | --- |
| RIP v2 | Distance Vector | Hop Count | Small Networks |
| OSPF | Link State | Cost | Large Networks |
| BGP | Path Vector | Path Attributes | Internet (Between AS) |

Memory Tricks

RIP

RIP = Route using Hop Count

Maximum Hop = 15

OSPF

OSPF = Open Shortest Path First

Metric = Cost

BGP

BGP = Between Global Providers

Used by ISPs

ICMP

ICMP = Internet Control Message Protocol

Used for Ping and Traceroute

One-Mark Questions

- Define Distance Vector Routing.

- What is RIPv2?

- What is Link State Routing?

- Define OSPF.

- Define BGP.

- What is ICMP?

- What is Hop Count?

- Expand OSPF.

- Expand BGP.

- Expand ICMP.

Two/Four-Mark Questions

- Explain Distance Vector Routing.

- Explain the working of RIPv2.

- Explain Link State Routing.

- Explain OSPF.

- Explain BGP.

- Explain ICMP.

- Differentiate between RIP and OSPF.

- Differentiate between Distance Vector and Link State Routing.

Viva Questions

- Which routing protocol uses Hop Count?

- Which routing protocol uses Cost?

- Which routing protocol is used by ISPs?

- What is the maximum hop count in RIP?

- Which protocol is used by the ping command?

- What is the purpose of ICMP?

Quick Revision

- Distance Vector Routing → Shares routing tables with neighbouring routers.

- RIPv2 → Distance Vector protocol, uses Hop Count, maximum 15 hops.

- Link State Routing → Routers maintain a complete network topology.

- OSPF → Link State protocol, uses Cost, suitable for large networks.

- BGP → Used between Autonomous Systems (AS), mainly by ISPs.

- ICMP → Used for error reporting and diagnostics (e.g., ping and tracert).

Unit 2 Summary

1. Router

A Router is a networking device that connects different networks and forwards data packets using IP addresses.

Functions

- Connects multiple networks

- Forwards packets

- Selects the best route

- Maintains routing tables

- Reduces network traffic

Router Architecture

- Input Ports – Receive incoming packets.

- Switching Fabric – Transfers packets to the correct output port.

- Output Ports – Send packets to the destination.

- Routing Processor – Maintains routing tables and runs routing protocols.

Key Point:
Router = Connects Networks

2. Routing Table

A Routing Table is a database inside a router that stores information about network routes.

Contains

- Destination Network

- Subnet Mask

- Next Hop

- Interface

- Metric (Cost)

Working

- Packet arrives.

- Router checks destination IP.

- Searches the routing table.

- Selects the best route.

- Forwards the packet.

Key Point:
Routing Table = Path Information

3. Queueing

Queueing is the temporary storage of packets when many packets arrive at the router simultaneously.

Advantages

- Prevents packet loss

- Maintains packet order

- Reduces congestion

Disadvantages

- May increase delay

- Queue overflow can cause packet loss

Key Point:
Queueing = Waiting Line for Packets

4. Switching

Switching is the process of transferring data from the source to the destination.

Types of Switching

Circuit Switching

- Dedicated communication path

- Used in telephone networks

Packet Switching

- Data divided into packets

- Packets travel independently

- Used on the Internet

Message Switching

- Entire message is stored and then forwarded

- Higher delay

5. Routing Protocols

A Routing Protocol is a set of rules that helps routers exchange routing information and find the best path.

Types

Static Routing

- Configured manually

- Used in small networks

Dynamic Routing

- Updates routes automatically

- Used in large networks

Examples:

- RIP

- OSPF

- BGP

6. Distance Vector Routing

- Routers share routing tables with neighboring routers.

- Uses Hop Count to measure distance.

Advantages

- Simple

- Easy to configure

Disadvantages

- Slow convergence

- Routing loops may occur

Key Point:
Distance Vector = Shares Routing Tables

7. RIP Version 2 (RIPv2)

RIP is a Distance Vector Routing Protocol.

Features

- Metric = Hop Count

- Maximum Hop Count = 15

- Hop Count 16 = Unreachable

- Updates every 30 seconds

Advantages

- Simple

- Automatic updates

Disadvantages

- Suitable only for small networks

- Slow convergence

Key Point:
RIP → Hop Count (Maximum = 15)

8. Link State Routing

Each router maintains a complete map of the network topology.

Working

- Discovers neighbors

- Creates Link State Advertisements (LSAs)

- Shares LSAs

- Calculates the shortest path

Advantages

- Fast convergence

- No routing loops

- Suitable for large networks

Key Point:
Link State = Complete Network Map

9. OSPF (Open Shortest Path First)

OSPF is a Link State Routing Protocol.

Features

- Metric = Cost

- Uses Dijkstra's Shortest Path Algorithm

- Fast convergence

- Suitable for large networks

Advantages

- Fast

- Reliable

- Scalable

Disadvantages

- Complex configuration

- Higher memory usage

Key Point:
OSPF → Cost Metric

10. BGP (Border Gateway Protocol)

BGP exchanges routing information between different Autonomous Systems (AS).

Autonomous System (AS)

A group of networks managed by one organization.

Examples:

- Internet Service Providers (ISPs)

- Universities

- Large organizations

Features

- Used on the Internet

- Connects Autonomous Systems

- Highly scalable

Key Point:
BGP = Used by ISPs

11. ICMP (Internet Control Message Protocol)

ICMP reports network errors and tests network connectivity.

Functions

- Reports errors

- Checks connectivity

- Helps troubleshoot network issues

Common Uses

- Ping – Tests whether a destination is reachable.

- Traceroute (tracert) – Shows the path packets take to reach the destination.

Key Point:
ICMP = Error Reporting and Diagnostics

Important Differences

Static Routing vs Dynamic Routing

| Static Routing | Dynamic Routing |
| --- | --- |
| Manual configuration | Automatic updates |
| Small networks | Large networks |
| No routing protocol | Uses routing protocols |

Distance Vector vs Link State Routing

| Distance Vector | Link State |
| --- | --- |
| Shares routing tables | Shares topology information |
| Slow convergence | Fast convergence |
| Less memory | More memory |
| Example: RIP | Example: OSPF |

RIP vs OSPF

| RIP | OSPF |
| --- | --- |
| Distance Vector | Link State |
| Metric = Hop Count | Metric = Cost |
| Small networks | Large networks |
| Slow convergence | Fast convergence |

Memory Tricks

- Router → Connects Networks

- Routing Table → Stores Routes

- Queueing → Packets Wait

- Switching → Transfers Data

- Distance Vector → Shares Routing Tables

- RIP → Hop Count (Max 15)

- Link State → Complete Network Map

- OSPF → Cost + Dijkstra Algorithm

- BGP → Between Autonomous Systems

- ICMP → Ping & Traceroute

Most Important MSBTE Exam Questions

2 Marks

- Define Router.

- Define Routing Table.

- What is Queueing?

- Define Switching.

- Define RIPv2.

- Define OSPF.

- Define BGP.

- Define ICMP.

4 Marks

- Explain Router Architecture.

- Explain Routing Table.

- Explain Queueing.

- Explain Distance Vector Routing.

- Explain Link State Routing.

- Explain ICMP.

6 Marks

- Explain RIPv2 with features.

- Explain OSPF with working.

- Explain BGP with features.

- Differentiate between RIP and OSPF.

- Differentiate between Distance Vector and Link State Routing.

Quick Revision

- Router → Connects different networks.

- Routing Table → Stores routing information.

- Queueing → Temporary storage of packets.

- Switching → Transfers data between devices.

- Distance Vector Routing → Exchanges routing tables with neighbours.

- RIPv2 → Distance Vector protocol using Hop Count (maximum 15).

- Link State Routing → Maintains the complete network topology.

- OSPF → Link State protocol using Cost.

- BGP → Connects different Autonomous Systems (AS), mainly used by ISPs.

- ICMP → Used for Ping, Traceroute, and error reporting.